SPECIFICATION FOR APPROVAL

CUSTOMER LINE

| HANDLING | CHECK | APPROVAL | CONFIRMATION |
| --- | --- | --- | --- |
| DATE: |  |  |  |

PLEASE SIGN AND RETURN ONE COPY.

With your signature, you confirm all the details in this specification, and the products will be made according to below description.

REMARKS：

CUSTOMER: SK4889

PRODUCT: Power Adaptor

DESIGNED NO: SK- W20190826002

MODEL NO: SK03T-1200150K

VERSION: 001

MODEL NUMBER: SK4889-1200150K1-01

深圳市森树强电子科技有限公司

Shenzhen Simsukian Electronics Technology Co., Ltd.

电话(TEL):0755-27488856	传真(FAX):0755-27488356

地址 ADD：深圳市龙华区观澜街道大坪社区佳怡工业园 5 号厂房

The 5th plant, Jiayi Industrial Park, Daping Community, Guanlan Street Longhua  Dist rict, Shenzhen P.R. China

| ITEM. | Revision Date | Revision Content | Effective Date | Arbiter | Revisor | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2019.08,26 | Initial | 2019.08.26 | 姚德允 | 钱亚苹 |  |
| MODEL NO. | SK03T-1200150K |  | Approve | Check | Preparation |  |
| DESIGN NO. | SK-W20190826002 |  |  |  | 钱亚苹 |  |
| Date: |  |  |  |  |  |  |

Index

- Description	4

- Electrical Specification	4

- Protection	5

- Reliability	5

- Environment	6

- Safety and EMI Requirement 	7

- Test Equipment List	8

- Mechanical Requirement	8

- Mechanical Specifications	9

- Weight	10

- Label Drawing	10

- Input/Output cable	11

- Package Drawin	12

1. Description

This specification apply to the Switching power whose model No. is SK03T-1200150K, which has a high efficiency and reliability, and have the over voltage, over current and short circuit function.

2. Electrical Specification

2.1 INPUT SPECIFICATION

| Input Voltage Range | 90 VAC – 264 VAC |
| --- | --- |
| Rated Voltage | 100 VAC / 240 VAC |
| Input Frequency Range | 47 Hz – 63 Hz |
| Rated Input Frenquency | 50 Hz / 60 Hz |
| Input Current | 0.6A Max Full Load, Rated Voltage |
| No Load Input Power | 0.1W Max No Loading, Rated Voltage |
| No Load Input Current | 60 mA Max No Loading, Rated Voltage |
| Power Factor | NA |
| Inrush Current | 100A Type Peak, 220 VAC Input, Cold Start |

2.2 OUTPUT SPECIFICATION

2.2.1 Rated Output

| Rated Output Voltage (VDC) | Output Voltage Range (VDC) | Output Ripple & Noise (mV) | Rated Output Current (mA) |
| --- | --- | --- | --- |
| 12 | 11.4 – 12.6 | ≤120 | 1500 |

Notes: The test condition as with oscilloscope with bandwidth 20MHz. A 10uF E-cap and 0.1uF ceramic-cap shall be connected to the connector in parallel._

2.2.2 Output Overshoot At Turn On & Turn Off

| Rated Output Voltage (VDC) | Proportion Of The Output Voltage Overshoot |  |
| --- | --- | --- |
|  | Turn On | Turn Off |
| 12 | 5% | 5% |

Note: The test condition as: All of output Current from Min to Max

2.2.3 Combine d Load / Line Regulation

| Rated Output Voltage (VDC) | Min. Loading | Rated Loading | Line Regulation | Load Regulation |
| --- | --- | --- | --- | --- |
| 15 | OA | 2A | ±3% | ±5% |

2.2.4 Turn on delay time

Max. 3 Second Max. at 115Vac input and output Max. load.

2.2.5 Rise time:

40mS Max. at 115Vac input and output Max load.

2.2.6 Hold up time:

5mS Min. at 115Vac input and output Min. Load.

2.2.7 Average Efficiency:

In the regular voltage, The output current is at 25%, 50%, 75% and full load cases, The average efficiency of ≧85.00% (warm up 30 minutes), Which meet the standards of energy efficiency level VI.

3. Protection Feature

3.1 Output Short circuit protection:

The power supply will be auto recovered after short circuit faults remove.

3.2	Over current Protection

The power supply will be auto recovered after over current faults remove.

4. Reliability Requirements

4.1. Burn-in

The power supply shall be burn-in for 2-4 Hours under normal input and 80% rated load a 40℃±5℃

4.2.	MTBF Qualification

The MTBF shall be at least 20,000 hours at 25℃, Full load and nominal input condition

5. Environment Requirement

5.1	Operating Temperature: 0℃-40℃

5.2	Storage Temperature: -40℃ to 85℃

5.3	Relative Humidity: 25%~90%, Relative Humidity 25%-90%,Non-Condensing

5.4	Storage Humidity: 10%~90%, Relative Humidity 10%-90%,Non-Condensing

5.5	Vibration:

5.5.1. Operating Standard: IEC 721-3-3 3M3

5 ~9Hz,A=1.5mm

9~200Hz, Acceleration 5m/S

5.5.2. Transportation: IEC721-3-2 2 2M2

5-9Hz, A=3.5mm,

9~200Hz, Acceleration=5m/S,

200~500Hz, Acceleration=15m/S

5.5.3. Axes, 10 cycles per axis:

No permanent damage may occur during testing and The product can restore to its original situation  after  power off / on.

5.6	Dropping Packed:

After the Dropping test (1 corner, 3 edges, and 6 surfaces, Height: 76cm 10times, function well.

6. Safety and EMI Requirement:

6.1	Safety: Accord with mark ■

| Certificates | Country | Standard |  |
| --- | --- | --- | --- |
| □ UL/CUL | USA | UL60065 |  |
| □ UL/CUL | USA | UL60950 |  |
| □ TUV/GS | EUROPE | EN60065 |  |
| □TUV/GS | EUROPE | UL60950 |  |
| □FCC | USA | CLASS B |  |
| □CE | EUROPE | EN60065 |  |
| □CE | EUROPE | EN60950 |  |
| □MEPS | AUSTRALIA | AS/NZS 4665 |  |
| □SAA | AUSTRALIA | AS/NZS 60065 |  |
| □SAA | AUSTRALIA | AS/NZS 60950 |  |
| □CCC | CHINA | GB8898 |  |
| □CCC | CHINA | GB4943 |  |
| □PSE | JAPAN | J60950(H22) |  |
| □CB | EUROPE | IEC60065 |  |
| □CB | EUROPE | IEC60950 |  |
| □C-TICK | AUSTRALIA | AS/NZS CISPR13:2004 |  |
| □EK/KC | KOREA | K60065 |  |
| ■EK/KC | KOREA | K60950 |  |

6.2 EMI STANDARD: Meets the Limits

6.2.1. Fcc class B rules

6.2.2. EN55032 class B rules

6.3	Dielectric Strength Testing: Dielectric strength to meet the requirements under the table

6.3.1 HI-POT TEST:

| ITEM | SPECIFICATION | REMARK |
| --- | --- | --- |
| Primary to Secondary | 3000VAC/5mA/60S | No arcing |
| Primary to P.G. | - | - |
| Secondary to P.G | - | - |

Note: Before testing, input L/N short circuit, make output positive and negative short circuit and connected.

6.3.2. 500Vdc/60Sec, isolation resistance 〉50M ohm

Note: at the test, input L/N short circuit, make output positive and negative short circuit.

7. Test Equipment List

| No. | Name Of Equipment | Manufacturer | Model |
| --- | --- | --- | --- |
| 1 | AC Power Source | Chroma | 6210 |
| 2 | Oscilloscope | Tektronix | TDS1002B |
| 3 | Electronic Load | TET | T3515 |
| 4 | Multimeter | FLUCK | 87Ⅲ |
| 5 | Dynamometer | Chroma | 2100 |
| 6 | Thermograph | LUTRON | Tw-902c |
| 7 | Plug Lead Bend Test Machine | LIANXIN | LX-817 |
| 8 | Salt Spraying Tester | LIANXIN | LX-8827B |
| 9 | Vibration Table | JINGJI | FZD-25 |
| 10 | Drop Test Bed | SELF-MADE |  |

8. MECHANICAL REQUIREMENT

8.1 Enclosure

The power supply size: L * W * H = 87.97mm * 42.47mm * 49mm

8.2	Input Connector

Two pin input plug.

9. MECHANICAL SPECIFICATIONS

9.1 Dimension

L * W * H = 87.97mm * 42.47mm * 49mm, as below

10. Weight

The weight of the power supply shall be about 174g

11.	Label Drawing

11.1 Label: as below

12 DC cable

12.1 Wire size chart: One end 08-A card, one end E1508 cold pressure terminal,  as below

12.2	DC Polarity as below

The red terminal is positive and the yellow terminal is negative)

12.3 Wire size chart: NA

13. Package Drawing: 100 PCS / CARTON

PACKING

Our own packing style will be adopted if no special packing required .

A Inner package CPE bag

B Outer package

Paper-box.

Size: 460×360×240mm, 4 × 9 card packaging 3 sheet 450X350mm

C Outer package notes include the information

Customer Name, LOT Number, Model No., Date, and so on.